Real-Time AI Handgun Detection and Threat Alert Platform

- Purpose and Overview

This project develops a real time handgun detection system that outputs both visual and auditory alerts alongside push notifications and emails with a security feed to identify potential weapon threats in order to ensure the safety of individuals in public spaces.

- System Architecture

The device is built on a Raspberry Pi 4 and uses state-based architecture to integrate a PIR Motion Sensor, a USB Webcam, a pretrained weapon detection AI Agent, a LCD Display, a Buzzer, a manual override push button, and LEDs.

- Operational Workflow

- Startup and Idle State: Upon boot, the blue LED flashes slowly and the LCD displays “Turning On.” Once booted, the blue LED remains solid and the LCD displays “Detecting Motion.”

- Motion Detection and Image Analysis: Once the PIR Motion Sensor detects movement, the LCD Displays “Motion Detected” with the USB Webcam taking a picture a sending it to the pretrained weapon detection AI Agent.

- No Handgun Detected: If no handgun was found, then the second green LED lights up and the LCD displays “No Handgun Detected,” with the system returning to the Motion Detecting state

- Handgun Detected: If a handgun was detected, then the second red LED and the buzzer turn on. A push notification is sent to the user’s smart phone using Pushover Notifications and the USB Webcam takes a 30 second video to be sent the user’s email. The LCD displays “Alert: Handgun Detected!” and will remain in this state until the user presses the manual override push button

4. Manual Override and Reset

The device incorporates a physical push button that serves as an immediate reset at any time. If pressed, all alerts and recordings are immediately suspended and the LCD displays “Resetting.” The device is then returned to the Motion Detecting State.

5. Status and Error Indication

The Blue LED indicates the device’s health. On startup, it flashes slowly. When in use without any errors, it remains solid. When any error occurs, it flashes quickly, with the LCD displaying which error occurred.

6. Implications

The real time AI Handgun detection and alert device should be implemented in public spaces where handguns remain a threat. For example, as the number of school shootings and violence increase each year, school districts around the nation can utilize this device in order to identify a threat before an even takes place. As a result, public spaces, particularly schools, and individuals will remain safe and protected from handgun threats, ultimately decreasing the number of violent attacks each year.

7. Future Improvements

Currently, the prototype is built on a breadboard and jumper wires. The next steps include fabricating a PCB and creating a 3-D enclosure to improve structure and usability. Additionally, future improvements include a higher reliability rate and replacing the Raspberry Pi with a microcontroller to decrease costs and offer the opportunity for mass production.